Helix Spin Dynamics

Mathematical calculation, R&D

Full Engineering Calculation Report:

25 Rafts × 8 Cars = 200 Cars.

1. Vehicle Capacity:

- Number of Rafts = 25

- Cars per Raft = 8

- Total Cars = 200

2. Car Dimension:

| Parameter | Value |
| --- | --- |
| Length | 5.5 m |
| Width | 2.5 m |
| Height | - m |

| Parking clearance: - Length = 6 m - Width = 2.5m - Height = 3 m |  |  |  |
| --- | --- | --- | --- |

3. One Raft Dimension:

| Parameter | Value |
| --- | --- |
| Diameter | 36 m |
| Height | 3 m |
| Parking Slots | 8 m |

4. Raft Calculation:

Raft Height:

25 × 3 = 75 m

Machine Room = 3 m

Foundation = 3 m

Total Underground Depth:

75 + 3 + 3 = 81 m

5. Land Requirement:

Diameter = 36 m

Area,

= π × 18²

= 1017.88 m²

Practical excavation and working clearance:

36 × 36 = 1296 m²

6. Car Load:

Design Load:

1 Car = 3 Ton

200 Cars

200 × 3

= 600 Ton

7. Steel Raft Weight:

Industrial Estimate:

1 Raft = 5 Ton

25 Rafts

25 × 5

= 125 Ton

8. Central Pillar:

Heavy Duty Structural Steel

≈ 50 Ton

9. Lift Platform:

Heavy Structural Platform

≈ 30 Ton

10. Bearings & Rotating Assembly:

25 Heavy Duty Slewing Bearings

≈ 20 Ton

11. Hydraulic Structure:

Cylinder Mount

Pipe

Frame

≈ 15 Ton

Total Moving Weight:

| Component | Weight |
| --- | --- |
| Cars | 600 Ton |
| Steel Rafts | 125 Ton |
| Central Pillar | 50 Ton |
| Lift Platform | 30 Ton |
| Bearings | 20 Ton |
| Hydraulic Structure | 15 Ton |
| Total | 840 Ton |

12. Safety Factor:

Industrial Safety Factor

= 1.5

Design Load:

840 × 1.5

= 1260 Ton

Round Off

= 1260 Ton

Hydraulic Calculation:

Hydraulic Cylinders

Quantity = 8

Total Load,

1260 Ton

Each Cylinder,

1260 ÷ 8

≈ 157.5 Ton

Recommended,

8 × 160 Ton Hydraulic Cylinders

Hydraulic Power Pack:

| Item | Quantity |
| --- | --- |
| Hydraulic Power Pack | 2 |
| Hydraulic Oil Tank | 2 |
| Hydraulic Pump | 4 (2 Working + 2 Standby) |
| Hydraulic Cylinders | 8 |
| Pressure Relief Valve | 2 |
| Direction Valve | 2 |
| Flow Control Valve | 8 |
| Check Valve | 8 |
| Pressure Sensor | 8 |
| Pressure Gauge | 4 |
| Oil Filter | 4 |
| Oil Cooler | 2 |
| PLC Controller | 1 |

Motor Calculation:

Each raft rotates independently.

≈ 25 Rafts

25 Geared Motors

Motor Rating ≈ 5 HP each

Total ≈ 125 HP

Electrical Calculation:

| Equipment | Power |
| --- | --- |
| Hydraulic Motors | 180 HP |
| 25 Rotation Motors | 125 HP |
| PLC | 2 HP |
| Lighting | 5 HP |
| Cameras | 2 HP |
| LCD | 1 HP |

Total,

≈315 HP

≈ 235 kW

Civil Calculation:

Excavation:

36 × 36 × 81

= 104,976 m³

Concrete:

≈ 15,000–18,000 m³

Bearing:

Each Raft,

Heavy Duty Slewing Bearing

Quantity,

≈ 25

IR Sensors:

Each Raft,

8 Slots

Total,

25 × 8

= 200 IR Sensors

Camera:

Entry,

≈ 3

Exit,

≈ 3

Total

≈ 6 Cameras

LCD:

Entry = 3

Exit = 3

Control Room = 1

Total,

≈ 7 LCD Displays

Servo Barrier:

Entry = 3

Exit = 3

Total

≈ 6 Servo Barriers

Total Mechanical Components

| Component | Quantity |
| --- | --- |
| Rafts | 25 |
| Cars | 200 |
| Hydraulic Cylinders | 8 |
| Hydraulic Power Packs | 2 |
| Hydraulic Pumps | 4 |
| Rotation Motors | 25 |
| Slewing Bearings | 25 |
| IR Sensors | 200 |
| Ultrasonic Sensors | 6 |
| Cameras | 6 |
| LCD Displays | 7 |
| Servo Barriers | 6 |
| PLC | 1 |